# 8-bit CPU Design Report

# 1. Introduction

This report documents the design and simulation of a simple 8-bit CPU. The CPU supports basic operations including ADD, SUB, MUL, DIV, LOAD, STORE, and LOAD_IMM with a 4x8-bit register file and 256x8-bit RAM. The design is implemented in Verilog and tested using QuestaSim.

# 2. Design Overview

The CPU is divided into the following modules:
- Top module: Integrates all components and controls data flow.
- Instruction Decoder: Decodes 8-bit instructions into control signals.
- Register File: 4 general-purpose 8-bit registers (R0-R3).
- ALU: Performs arithmetic operations (ADD, SUB, MUL, DIV).
- RAM: 256x8-bit memory supporting load and store operations.

# 3. Instruction Format

Each instruction is 8 bits:
- [7:4] Opcode (4 bits)
- [3:2] Destination register (rd)
- [1:0] Source register or immediate (rs2/imm)

Supported Opcodes:
- 0001: ADD
- 0010: SUB
- 0101: LOAD
- 0110: STORE
- 0111: LOAD_IMM

# 4. Simulation Results

A testbench was created to verify the CPU functionality. The following outputs were observed:

| Time (ns) | Instruction | Registers Written | ALU Operation | RAM Operation |
| --- | --- | --- | --- | --- |
| 20 | LOAD_IMM R0,1 | R0=1 | - | - |
| 30 | LOAD_IMM R1,2 | R1=2 | - | - |
| 40 | ADD R2,R0,R1 | R2=3 | a=1 b=2 result=3 | - |
| 50 | STORE R2,R0 | - | - | ADDR=0 DATA=3 |
| 60 | LOAD R3,0 | R3=3 | - | ADDR=0 DATA=3 |

# 5. Final Register and Memory State

Registers:
R0=1, R1=2, R2=3, R3=3
RAM[0]=3

# 6. README

Files included:
- top.v: Top module integrating all components.
- instr_decoder.v: Instruction decoder module.
- alu.v: Arithmetic logic unit.
- reg_file.v: 4x8-bit register file.
- ram.v: 256x8-bit RAM module.
- top_tb.v: Testbench for simulation.

To simulate:
1. Compile all Verilog files in QuestaSim.
2. Run the testbench top_tb.v.
3. Observe the console output for register and RAM operations.
4. Use wave.do for waveform analysis (if available).

The testbench demonstrates data path functionality including register writes, ALU operations, memory load/store, and immediate loading.